# Lab 04 – Worksheet

| Name: Ahmad Tariq | ID: am10770 | Section: T4 |
| --- | --- | --- |

Note: Assumptions and logics should be explained separately in tasks after the task results.

## Task 1

Provide appropriately commented codes.

Task 1a) .text .globl main main: addi x10, x0, 3 jal x1, fact addi x11, x10, 0 li x10, 1 ecall j end fact: addi sp, sp, -8 sw x1, 4(sp) sw x10, 0(sp) addi x5, x10, -1 bge x5, x0, L1 addi x10, x0, 1 addi sp, sp, 8 jalr x0, 0(x1) L1: addi x10, x10, -1 jal x1, fact addi x6, x10, 0 lw x10, 0(sp) lw x1, 4(sp) addi sp, sp, 8 mul x10, x10, x6 jalr x0, 0(x1) end: j end Task 1b) .text .globl main main: li x10, 3 li x13, 1 loop: beq x10, x0, done mul x13, x13, x10 addi x10, x10, -1 beq x0, x0, loop done: addi x10, x13, 0 addi x11, x10, 0 li x10, 1 ecall end: j end

*Add snip of relevant Registers / Memory locations where you think results are obtained. Make sure the irrelevant area of snip is cropped. Make sure the irrelevant area of snip is cropped.

Task1a) Task1b)

Comments

Task1a: computes n! (factorial) recursively,each call pushes the return address and current n onto the stack (2 words), recurses down to the base case, then multiplies results back together while unwinding. Task 1b: computes n! iteratively using a while loop (accumulator × n, decrementing n each pass). No function calls are made, so no stack frame is needed at all, sp stays constant throughout. Stack does not get populated because there are no sp, sw, or lw instructions used anywhere which is also why no frame is needed. If we compare with task 1a, it pushes 2 words (return address + n) per call, Task 1b uses 0 bytes of stack, since it's a single non-recursive loop. Iterative is more memory-efficient and avoids stack overflow risk for large n.

## Task 2

Provide appropriately commented codes

.text .globl main main: addi x10, x0, 5 addi x18, x0, 2 jal x1, nitri addi x11, x10, 0 addi x10, x0, 1 ecall beq x0, x0, end nitri: blt x10, x18, basecase addi sp, sp, -8 sw x10, 0(sp) sw x1, 4(sp) addi x10, x10, -1 jal x1, nitri addi x19, x10, 0 lw x1, 4(sp) lw x10, 0(sp) addi sp, sp, 8 add x10, x19, x10 jalr x0, 0(x1) basecase: addi x10, x0, 1 jalr x0, 0(x1) end: j end

Add screenshot of your results

Comments

Task 2: Computes the nth triangular number (1+2+...+n) recursively, structurally identical to Task 1a but adding instead of multiplying at each unwind step.

## Task 3

## Provide appropriately commented codes

.text .globl main main: addi x11, x0, 5 addi x10, x0, 0x100 addi x5, x0, 1 sw x5, 0(x10) addi x5, x0, 8 sw x5, 4(x10) addi x5, x0, 2 sw x5, 8(x10) addi x5, x0, 6 sw x5, 12(x10) addi x5, x0, 5 sw x5, 16(x10) beq x10, x0, end beq x11, x0, end bubble_sort: addi x20, x0, 0 bigloop: bge x20, x11, end addi x21, x20, 0 smallloop: bge x21, x11, outerloopnext slli x22, x20, 2 add x22, x22, x10 lw x24, 0(x22) slli x23, x21, 2 add x23, x23, x10 lw x25, 0(x23) bge x24, x25, swapnotdone sw x25, 0(x22) sw x24, 0(x23) swapnotdone: addi x21, x21, 1 beq x0, x0, smallloop outerloopnext: addi x20, x20, 1 beq x0, x0, bigloop end: j end

Add screenshots of your results

Before sort: After being sort:

Comments:

Task 3: Sorts an array in memory using bubble sort (nested loops comparing and swapping elements). Unlike the other tasks, it modifies memory directly rather than computing a single return value, and produces no printed output.

## Task 4

## Provide appropriately commented codes

.text .globl main main: addi x10, x0,5 jal x1, fibonacci addi x11, x10, 0 addi x10, x0, 1 ecall beq x0, x0, end fibonacci: addi x5, x10, -2 bge x5, x0, recursive jalr x0, 0(x1) recursive: addi sp, sp, -12 sw x1, 8(sp) sw x10, 4(sp) addi x10, x10, -1 jal x1, fibonacci sw x10, 0(sp) lw x10, 4(sp) addi x10, x10, -2 jal x1, fibonacci lw x6, 0(sp) lw x1, 8(sp) addi sp, sp, 12 add x10, x10, x6 jalr x0, 0(x1) end: j end

Add screenshots of your results

Comments:

Task 4: Computes the nth Fibonacci number recursively. Unlike Tasks 1a and 1b, each call makes two recursive calls (fib(n-1) and fib(n-2)), requiring 3 stack words instead of 2 to hold the extra intermediate result.

https://github.com/Ahmaddddd7/CA_LAB

# Lab 04 – Nest Procedures and Sorting

# Assessment Rubrics

# Points Distribution

| Task No. | LR2 Code | LR 5 Results | AR 7 Report Submission & Github |
| --- | --- | --- | --- |
| Task 1 | /10 | /05 | /10 & /10 |
| Task 2 | /10 | /10 |  |
| Task 3 | /10 | /10 |  |
| Task 4 | /15 | /10 |  |
| Total Points | /100 Points |  |  |
| CLO Mapped | CLO 2 |  |  |

For description of different levels of the mapped rubrics, please refer the provided Lab Evaluation Assessment Rubrics and Affective Domain Assessment Rubrics.

| # | Assessment Elements | Level 1: Unsatisfactory Points 0-1 | Level 2: Developing Points 2 | Level 3:Good Points 3 | Level 4:Exemplary Points 4 |
| --- | --- | --- | --- | --- | --- |
| LR2 | Program/Code/ Simulation Model/ Network Model | Program/code/simulation model/network model does not implement the required functionality and has several errors. The student is not able to utilize even the basic tools of the software. | Program/code/simulation model/network model has some errors and does not produce completely accurate results. Student has limited command on the basic tools of the software. | Program/code/simulation model/network model gives correct output but not efficiently implemented or implemented by computationally complex routine. | Program/code/simulation /network model is efficiently implemented and gives correct output. Student has full command on the basic tools of the software. |
| LR5 | Results & Plots | Figures/ graphs / tables are not developed or are poorly constructed with erroneous results. Titles, captions, units are not mentioned. Data is presented in an obscure manner. | Figures, graphs and tables are drawn but contain errors. Titles, captions, units are not accurate. Data presentation is not too clear. | All figures, graphs, tables are correctly drawn but contain minor errors or some of the details are missing. | Figures / graphs / tables are correctly drawn and appropriate titles/captions and proper units are mentioned. Data presentation is systematic. |
| AR9 | Report | All the in-lab tasks are not included in report and / or the report is submitted too late. | Most of the tasks are included in report but are not well explained. All the necessary figures / plots are not included. Report is submitted after due date. | Good summary of most the in-lab tasks is included in report. The work is supported by figures and plots with explanations. The report is submitted timely. | Detailed summary of the in-lab tasks is provided. All tasks are included and explained well. Data is presented clearly including all the necessary figures, plots and tables. |