Lab 1 - Getting Started with RISC-V (Assembly Language) in VS Code

| Name: Ahmad Tariq | Student ID: am10770 |
| --- | --- |

Task 3:

Code:

.text

.globl main

main:

li x1, 5

li x2, 0         #a = x1

addi x2, x2, 0      #b = x2

addi x1, x2, 32

add x3, x1, x2           #d = x3

addi x3, x3, -5

sub x4, x1, x3             #e = x4

sub x5, x2, x1                    #(b-a) = x5

add x4, x4, x5

add x4, x4, x3

add x4, x1, x4

add x4, x4, x2

add x4, x4, x3

end:

j end

Results:

This code translates a sequence of C variable assignments and arithmetic operations (involving a, b, d, e) into RISC-V. Since RISC-V has no concept of variables, each C statement is mapped to specific registers, and intermediate results are computed step by step using temporary registers before being combined into the final values.

Task 4a:

- Store x10 as unsigned integer at address 0x100.

li x10, 0x78786464

li x2, 0x100

sw x10, 0(x2)

- Store x11 as unsigned integer at address 0x1F0.

li x3, 0x1F0

sw x11, 0(x3)

- Load an unsigned short integer (two bytes) from address 0x100 in x12.

lhu x12, 0(x2)

- Load a short integer from address 0x1F0 in register x13.

lh x13, 0(x3)

- Load a singed character from address 0x1F0 in register x14.

lb x14, 0(x3)

Demonstrates RISC-V memory instructions by manually storing two values (in x10 and x11) into memory at specific addresses using sw, then reading them back using different load instructions (lhu, lh, lb) to show how signedness and data size (unsigned short, signed short, signed byte) affect how the same memory bytes are interpreted.

Task 4b:

Code:

.text

.globl main

main:

li x1, 0x100

li x2, 0x200

li x3, 0x300

li x5, 10

sb x5, 0(x1)        #a[0] = 10

li x5, 20

sb x5, 1(x1)        #a[1] = 20

li x5, 30

sb x5, 2(x1)        #a[2] = 30

li x5, 40

sb x5, 3(x1)        #a[3] = 40

li x6, 10

sh x6, 0(x2)        #b[0] = 10

li x6, 20

sh x6, 2(x2)        #b[1] = 20

li x6, 30

sh x6, 4(x2)        #b[2] = 30

li x6, 40

sh x6, 6(x2)        #b[3] = 40

lb x20, 0(x1)

lh x21, 0(x2)

add x22, x20, x21

sw x22, 0(x3)

lb x20, 1(x1)

lh x21, 2(x2)

add x22, x20, x21

sw x22, 4(x3)

lb x20, 2(x1)

lh x21, 4(x2)

add x22, x20, x21

sw x22, 8(x3)

lb x20, 3(x1)

lh x21, 6(x2)

add x22, x20, x21

sw x22, 12(x3)

end:

j end

Results:

Computes c[i] = a[i] + b[i] for 4 array elements, but writes it out

manually instead of using a loop. Since a holds chars (1 byte), b holds

shorts (2 bytes), and c holds unsigned ints (4 bytes), each array needs

different offsets to reach the correct element The code loads one element

from a and b, adds them, and stores the result in c which is

repeated four times with the correct offsets each time.

Assessment Rubric

Lab 1: Getting Started with RISC-V (Assembly Language) n VS Code

| Name Ahmad Tariq | Student ID: am10770 | Section: T4 |
| --- | --- | --- |

Points Distribution:

|  | Task No. | LR 2 (Code) | LR 5 (Results) |
| --- | --- | --- | --- |
| In-Lab | Task 1 | - | /15 |
|  | Task 2 | - | /15 |
|  | Task 3 | /10 | /5 |
|  | Task 4a | /10 | /5 |
|  | Task 4b | /10 | /10 |
| Total Points: 100 |  | /30 | /50 |
| CLO Mapped |  | CLO 2 |  |

| Affective Domain Rubric |  | Points | CLO Mapped |
| --- | --- | --- | --- |
| AR7 | Report Submission & Git Upload | /10 & /10 | CLO 2 |

| CLO | Total Points | Points Obtained |
| --- | --- | --- |
| 2 | 100 |  |
| Total | 100 |  |