# Lab 03 – Worksheet

| Name: Ahmad Tariq | ID: 10770 | Section: T4 |
| --- | --- | --- |

Note: Assumptions and logics should be explained separately in tasks after the task results.

## Task 1

Provide appropriately commented codes (Mention question part before each part)

.text .globl main main: addi x10,x0,12 addi x11,x0,12 jal x1,sum addi x11, x10, 0 li x10, 1 ecall j exit sum: add x10, x11, x10 jalr x0, 0(x1) exit: end: j end

Add screenshot of your results (register)

PC before jal statement: PC after jal statement: PC before jalr statement: PC after jalr statement: main puts 12 in x10 and 12 in x11, then jal x1, sum saves the return address (0x0C) in x1 and jumps to sum. sum adds x11 and x10 into x10 (24) and returns with jalr. Back in main, the result is copied to x11, x10 is set to 1 (the print-integer ecall code), and ecall prints 24. The program then ends in an infinite loop.

## Task 2

Provide appropriately commented codes (Mention question part before each part)

.text .globl main main: li x10, 4 li x11, 3 li x12, 2 li x13, 1 jal x1, longleaf j end longleaf: addi sp,sp,-12 sw x18, 8(sp) sw x19,4(sp) sw x20, 0(sp) add x18, x10, x11 add x19, x12, x13 sub x20, x18, x19 addi x10, x20, 0 lw x18, 8(sp) lw x19, 4(sp) lw x20, 0(sp) addi sp, sp, 12 jalr x0, 0(x1) end: j end

Add screenshot of your results

PC Before jal statement: PC after jal statement: PC before jalr statement: PC after jalr statement: a procedure that computes (g + h) − (i + j). Main loads g = 4, h = 3, i = 2 and j = 1 into x10 to x13 and calls longleaf. The procedure needs x18 to x20 as scratch registers. They are saved registers, so it first pushes them onto the stack (sp goes down by 12). It then computes x18 = 4 + 3 = 7, x19 = 2 + 1 = 3 and x20 = 7 − 3 = 4, and moves 4 into x10 as the return value. Finally it restores x18 to x20, pops the stack and returns. The result is 4 in x10.

## Task 3 Provide appropriately commented codes (Mention question part before each part)

.text .globl main main: addi sp, sp, -8 li x5, 10 sw x5, 0(sp) li x5, 20 sw x5, 4(sp) mv x10, sp li x11, 0 jal x1, swap lw x12, 0(sp) lw x13, 4(sp) addi sp, sp, 8 end: j end swap: addi sp,sp,-12 sw x5, 8(sp) sw x6, 4(sp) sw x7, 0(sp) slli x6, x11, 2 add x6, x10, x6 lw x5, 0(x6) lw x7, 4(x6) sw x7, 0(x6) sw x5, 4(x6) lw x7, 0(sp) lw x6, 4(sp) lw x5, 8(sp) addi sp, sp, 12 jalr x0, 0(x1)

Add screenshots of your results

Before swap: After swap: PC Before jal statement: PC after jal statement: PC before jalr statement: PC after jalr statement: swaps two array elements. Main builds a 2-element array on the stack (10 and 20), passes its address in x10 and the index k = 0 in x11, and calls swap. Swap saves x5 to x7, then computes the address of v[k] as base + k × 4 (the slli by 2 scales k to a byte offset). It loads v[k] and v[k+1] and stores them back crossed over, so the array becomes 20, 10. It then restores the registers and returns. Main reads the array back into x12 = 20 and x13 = 10 to confirm

## Task 4 Provide appropriately commented codes

.text .globl main main: addi sp, sp, -8 addi x5, x0, 78 sb x5, 0(sp) addi x5, x0, 111 sb x5, 1(sp) sb x0, 2(sp) addi x11, sp, 0 addi x10, sp, 4 jal x1, strcpy lb x12, 4(sp) lb x13, 5(sp) lb x14, 6(sp) addi sp, sp, 8 end: j end strcpy: addi sp, sp, -4 sw x19, 0(sp) addi x19, x0, 0 loop: add x5, x11, x19 lb x6, 0(x5) add x7, x10, x19 sb x6, 0(x7) beq x6, x0, leave addi x19, x19, 1 jal x0, loop leave: lw x19, 0(sp) addi sp, sp, 4 jalr x0, 0(x1)

Add screenshots of your results

Before getting copied: After being copied: Before jal statement: After jal statement: Before jalr statement: After jalr statement: strcpy, which copies a string including the null terminator. Main stores "No" (78, 111, then a 0 terminator) at sp, points x11 (source y) at it, and points x10 (destination x) at the space 4 bytes after it. Strcpy saves x19 and uses it as the index i. Each pass loads the byte y[i], stores it to x[i], and exits if the byte was 0. Otherwise it adds 1 to i and loops. The copy happens before the test, so the null terminator is copied too. It runs 3 passes, restores x19 and returns. x12 = 78, x13 = 111 and x14 = 0 confirm the copy. Git link: https://github.com/Ahmaddddd7/CA_LAB

# Lab 03 – Introduction to RISC V Assembly (Jumps and Returns)

# Assessment Rubrics

# Points Distribution

| Task No. | LR2 Code | LR5 Results | AR7 Report Submission and Git Upload |
| --- | --- | --- | --- |
| Task 1 | /10 | /05 | /10 & /10 |
| Task 2 | /10 | /10 |  |
| Task 3 | /10 | /10 |  |
| Task 4 | /15 | /10 |  |
| Total Points | /100 Points |  |  |
| CLO Mapped | CLO 2 |  |  |

For description of different levels of the mapped rubrics, please refer the provided Lab Evaluation Assessment Rubrics and Affective Domain Assessment Rubrics.

| # | Assessment Elements | Level 1: Unsatisfactory Points 0-1 | Level 2: Developing Points 2 | Level 3:Good Points 3 | Level 4:Exemplary Points 4 |
| --- | --- | --- | --- | --- | --- |
| LR2 | Program/Code/ Simulation Model/ Network Model | Program/code/simulation model/network model does not implement the required functionality and has several errors. The student is not able to utilize even the basic tools of the software. | Program/code/simulation model/network model has some errors and does not produce completely accurate results. Student has limited command on the basic tools of the software. | Program/code/simulation model/network model gives correct output but not efficiently implemented or implemented by computationally complex routine. | Program/code/simulation /network model is efficiently implemented and gives correct output. Student has full command on the basic tools of the software. |
| LR5 | Results & Plots | Figures/ graphs / tables are not developed or are poorly constructed with erroneous results. Titles, captions, units are not mentioned. Data is presented in an obscure manner. | Figures, graphs and tables are drawn but contain errors. Titles, captions, units are not accurate. Data presentation is not too clear. | All figures, graphs, tables are correctly drawn but contain minor errors or some of the details are missing. | Figures / graphs / tables are correctly drawn and appropriate titles/captions and proper units are mentioned. Data presentation is systematic. |
| AR9 | Report | All the in-lab tasks are not included in report and / or the report is submitted too late. | Most of the tasks are included in report but are not well explained. All the necessary figures / plots are not included. Report is submitted after due date. | Good summary of most the in-lab tasks is included in report. The work is supported by figures and plots with explanations. The report is submitted timely. | Detailed summary of the in-lab tasks is provided. All tasks are included and explained well. Data is presented clearly including all the necessary figures, plots and tables. |