Lab 2 - Implementing Decision Instructions in RISC V Assembly Language

| Name:Ahmad Tariq | Student ID: 10770 |
| --- | --- |

Task 1:

Code:

Listing 3:

.text

.globl main

main:

li x20, 4

li x21, 3

li x22, 2

li x23, 1

bne x22, x23, else

add x19, x20, x21

beq x0, x0, exit

else:

sub x19, x20, x21

exit:

end:

j end

Implements an if/else statement in RISC-V, where f = g + h if i == j, otherwise f = g - h. It uses bne to check whether i and j are unequal and branch to the else case, with an unconditional jump used to skip over the else block when the if-condition is true mimicking how a compiled if/else avoids falling through both branches.

Listing 4:

.text

.globl main

main:

li x22, 0

li x24, 2

li x25, 0x100

li x1, 2

sw x1, 0(x25)

li x2, 2

sw x2, 4(x25)

li x3, 4

sw x3, 8(x25)

li x4, 2

sw x4, 12(x25)

Loop:

slli x10, x22 , 2

add x10, x10, x25

lw x9, 0(x10)

bne x24, x9, exit

addi x22, x22, 1

beq x0, x0, Loop

exit:

end:

j end

Implements a while loop that searches an array for the first value different from a target k. It stores test values into memory, then loops through each element, computing its address, loading the value, and comparing it to k. The loop continues (using an unconditional jump back to Loop) as long as the values match, and exits as soon as it finds one that doesn't, leaving the index of that "odd" value in the loop counter register.

Results:

Listing 3:

Before conditional jump:

After conditional jump:

Before unconditional jump:

After unconditional jump:

Listing 4:

Before conditional jump:

After conditional jump:

Before unconditional jump:

After unconditional jump:

Memory:

Task 2:

Code:

.text

.globl main

main:

li x20, 4

li x21, 2

li x22, 3

li x23, 4

li x24, 1

li x25, 2

li x26, 3

li x27, 4

beq x20, x24, case1

beq x20,x25, case2

beq x20, x26, case3

beq x20, x27, case4

beq x0, x0, default

case1:

add x21, x22, x23

beq x0,x0, exit

case2:

sub x21, x22, x23

beq x0, x0, exit

case3:

slli x21, x22, 1

beq x0, x0, exit

case4:

srli x21, x22, 1

beq x0, x0, exit

default:

li x21, 0

beq x0, x0, exit

exit:

end:

j end

Implements a switch statement in RISC-V, comparing variable x against each possible case value (1 through 4) using a chain of beq instructions, and falling through to a default case if none match. Each case performs a different arithmetic operation on b and c to compute a (addition, subtraction, multiply-by-2, divide-by-2), with unconditional jumps ensuring only one case's result is kept before exiting.

Results:

Before conditional jump:

After conditional jump:

Before unconditional jump:

After unconditional jump:

Task 3:

Code:

.text

.globl main

main:

li x22, 0

li x23, 0

li x2, 0x200

li x3, 10

for_loop1:

beq x22, x3, end_for1

slli x4, x22, 2

add x5, x4, x2          #a[i]

sw x22, 0(x5)

addi x22, x22, 1

beq x0,x0,for_loop1

end_for1:

li x22, 0

li x23, 0

for_loop2:

beq x22, x3, end_for2

slli x4, x22, 2

add x5, x4, x2           #a[i]

lw x7, 0(x5)

add x23, x23, x7

addi x22, x22, 1

beq x0,x0,for_loop2

end_for2:

end:

j end

Implements two sequential for-loops over a 10-element integer array. The first loop populates a[i] = i for each index, storing values directly into memory. The second loop then reads each element back with lw and accumulates them into a running total (sum), effectively computing the sum of 0 through 9 using array-based memory access instead of direct register arithmetic.

Results:

Before unconditional jump:

After unconditional jump:

Before conditional jump:

After conditional jump:

Task 4:

Code:

.text

.globl main

main:

li x5, 10       # a

li x6, 10       # b

li x7, 0        # i

li x29, 0       # j

li x10, 0x100   #base address of array D

for_loop1:

beq x7, x5, end_for1

li x29, 0

for_loop2:

beq x29, x6, end_for2

slli x8, x29, 4

add x11, x10, x8

add x12, x7, x29

sw x12, 0(x11)

addi x29, x29, 1

beq x0,x0,for_loop2

end_for2:

addi x7, x7, 1

beq x0, x0, for_loop1

end_for1:

end:

j end

Implements a nested for-loop that computes D[4*j] = i+j for a 10×10 range of i and j values, writing results into a word array in memory. The inner loop resets j to 0 and iterates through all j values for each i, computing the correct byte offset (scaled by 16, since each array slot spans 4 elements × 4 bytes), storing i + j at that address with the outer loop incrementing i and repeating until both counters reach their limits.

Results:

Before conditional jump:

After conditional jump:

Before unconditional jump:

After unconditional jump:

Memory:

https://github.com/Ahmaddddd7/CA_LAB

Assessment Rubric

Lab 2 - Implementing Decision Instructions in RISC V Assembly Language

| Name Ahmad Tariq | Student ID: 10770 | Section: T4 |
| --- | --- | --- |

Points Distribution:

|  | Task No. | LR 2 (Code) | LR 5 (Results) |
| --- | --- | --- | --- |
| In-Lab | Task 1 | - | /5 |
|  | Task 2 | /10 | /10 |
|  | Task 3 | /15 | /10 |
|  | Task 4 | /20 | /10 |
| Total Points: 100 |  | /45 | /35 |
| CLO Mapped |  | CLO 2 |  |

| Affective Domain Rubric |  | Points | CLO Mapped |
| --- | --- | --- | --- |
| AR7 | Report Submission & Git Upload | /10 & /10 | CLO 2 |

| CLO | Total Points | Points Obtained |
| --- | --- | --- |
| 2 | 100 |  |
| Total | 100 |  |