# Lab 06 – Worksheet

| Name: Mohammad Jazib Javed / Ahmad Tariq | ID: 11972/ 10770 | Section: T4 |
| --- | --- | --- |

Note: Assumptions and logics should be explained separately in tasks after the task results.

## Task 1

Block Diagrams

1 bit ALU block diagram: 32 bit ALU block diagram:

Comments

ALUControl; Operation; Result. 0000; ADD; A + B. 0001; SUB; A − B. 0010; AND; A & B. 0011; OR; A ¦ B. 0100; XOR; A ^ B. 0101; SLL; A << B[4:0]. 0110; SRL; A >> B[4:0]. 0111; BEQ; A − B, with Zero = 1 when A == B.

## Task 2

Modules + Testbench:

MODULE: `timescale 1ns / 1ps ////////////////////////////////////////////////////////////////////////////////// // Company: // Engineer: // // Create Date: 09/30/2026 02:13:27 PM // Design Name: // Module Name: ALU // Project Name: // Target Devices: // Tool Versions: // Description: // // Dependencies: // // Revision: // Revision 0.01 - File Created // Additional Comments: // ////////////////////////////////////////////////////////////////////////////////// module ALU( input [31:0] A, // Operand A input [31:0] B, // Operand B input [3:0] ALUControl, // Selects which operation to perform output reg [31:0] ALUResult, // Result of the ALU output Zero // high when the result is 0 ); always @(*) begin //Used for combinational logic (creates its own sensitivity list) case (ALUControl) 4'b0000: ALUResult = A + B; // ADD 4'b0001: ALUResult = A - B; // SUB 4'b0010: ALUResult = A & B; // AND 4'b0011: ALUResult = A ¦ B; // OR 4'b0100: ALUResult = A ^ B; // XOR 4'b0101: ALUResult = A << B[4:0]; // SLL: shift A left by B[4:0] bits 4'b0110: ALUResult = A >> B[4:0]; // SRL: shift A right by B[4:0] bits 4'b0111: ALUResult = A - B; // BEQ: subtract, Zero = 1 if A == B default: ALUResult = 32'b0; // unused codes output 0 endcase end assign Zero = (ALUResult == 32'b0); // Zero flag: high when every bit of the result is 0 endmodule TestBench: `timescale 1ns / 1ps ////////////////////////////////////////////////////////////////////////////////// // Company: // Engineer: // // Create Date: 09/30/2026 02:19:14 PM // Design Name: // Module Name: ALU_tb // Project Name: // Target Devices: // Tool Versions: // Description: // // Dependencies: // // Revision: // Revision 0.01 - File Created // Additional Comments: // ////////////////////////////////////////////////////////////////////////////////// module ALU_tb(); reg [31:0] A, B; reg [3:0] ALUControl; wire [31:0] ALUResult; wire Zero; ALU dut (.A(A), .B(B), .ALUControl(ALUControl), .ALUResult(ALUResult), .Zero(Zero)); initial begin A = 32'hAAAAAAAA; B = 32'h55555555; ALUControl = 4'b0000; #10; // ADD = FFFFFFFF ALUControl = 4'b0001; #10; // SUB = 55555555 ALUControl = 4'b0010; #10; // AND = 00000000, Zero = 1 ALUControl = 4'b0011; #10; // OR = FFFFFFFF ALUControl = 4'b0100; #10; // XOR = FFFFFFFF A = 32'h00000001; B = 32'h00000004; ALUControl = 4'b0101; #10; // SLL = 00000010 A = 32'h80000000; B = 32'h0000001F; ALUControl = 4'b0110; #10; // SRL = 00000001 A = 32'h12345678; B = 32'h12345678; ALUControl = 4'b0111; #10; // BEQ equal = 00000000, Zero = 1 B = 32'h12345679; ALUControl = 4'b0111; #10; // BEQ not equal = FFFFFFFF, Zero = 0 $finish; end endmodule

Results:

## Task 3

## RTL Schematics

## `timescale 1ns / 1ps ## ////////////////////////////////////////////////////////////////////////////////// ## // Company: ## // Engineer: Muddassir Ali ## // Module Name: top_fsm_system ## // Project Name: Counter ## // Target Devices: Baasys 3 ## // ## ////////////////////////////////////////////////////////////////////////////////// ## module top_fsm_system ( ## input wire clk, ## input wire pbin, ## input wire [15:0] physical_sw, ## output wire [15:0] physical_leds ## ); ## // DEBOUNCER (Cleans up the physical reset button signal) ## wire rst_clean; ## wire [31:0] switch_data; // hold the value read from the switches ## reg [31:0] led_write_data = 32'd0; // counter value here ## wire slow_clk; ## debouncer rst_db ( ## .clk(clk), ## .pbin(pbin), ## .pbout(rst_clean) //generated a clean signal ## ); ## leds switch_reader ( ## .clk(clk), .rst(rst_clean), ## .btns(16'd0), // Not used for this FSM ## .writeData(32'd0), // We don't write to switches ## .writeEnable(1'b0), // Disabled ## .readEnable(1'b1), // Always ON so we can monitor switches ## .memAddress(30'd0), ## .switches(physical_sw), // Plug in the physical switches ## .readData(switch_data) // output data ## ); ## switches led_writer ( ## .clk(clk), .rst(rst_clean), ## .writeData(led_write_data), ## .writeEnable(1'b1), // Always ON so LEDs update instantly ## .readEnable(1'b0), .memAddress(30'd0), ## .readData(), // Ignored ## .leds(physical_leds) ## ); ## // clock_divider ticker ( ## // .clk_in(clk), // Feed it the 100MHz fast clock ## // .rst(rst_clean), // Feed it the clean reset signal ## // .clk_out(slow_clk) // It spits out the 1Hz slow clock! ## // ); ## // YOUR FSM AND COUNTER LOGIC ## // YOUR CODE HERE ## // Registers that hold the inputs read by the FSM ## reg [3:0] A_reg = 0; //stores operand A ## reg [3:0] B_reg = 0; //stores operand B ## reg [3:0] ctl_reg = 0; // stores the operation code ## // ALU (4-bit inputs zero-extended to 32 bits) ## wire [31:0] ALUResult; //32 bit result coming out of the ALU ## wire Zero; //Goes to 1 when result is 0 ## ALU alu ( ## .A({28'd0, A_reg}), //taking 4 bit input for A so zero extend to 32 bits ## .B({28'd0, B_reg}), //taking 4 bit input for B so zero extend to 32 bits ## .ALUControl(ctl_reg), ## .ALUResult(ALUResult), ## .Zero(Zero) ## ); ## // FSM: READ switches -> COMPUTE -> SHOW on LEDs -> repeat ## localparam S_READ = 2'd0, //State 0 : Read the switches (Get A,B And ALUcontrol values) ## S_COMPUTE = 2'd1, //State 1 : ALU performs operation ## S_SHOW = 2'd2; //State 2 : Display ALU result on LEDs ## reg [1:0] state = S_READ; // Register holding the current state (starts at State 0) ## always @(posedge clk) begin ## if (rst_clean) begin ## state <= S_READ; //reset:go back to the first state ## led_write_data <= 32'd0; // turns all leds off when reset ## end ## else begin ## case (state) ## S_READ: begin ## A_reg <= switch_data[3:0]; //save A from switches ## B_reg <= switch_data[7:4]; //save B from switches ## ctl_reg <= switch_data[11:8]; //save operation code ## state <= S_COMPUTE; //move to next state (compute) ## end ## S_COMPUTE: begin ## state <= S_SHOW; // wait one cycle for the ALU (using nonblocking assignment) ## end ## S_SHOW: begin ## // LED15 = Zero signal display ## // LED14-LED0 = lower 15 bits of the result ## led_write_data <= {16'd0, Zero, ALUResult[14:0]}; ## state <= S_READ; // goes back to read state ## end ## default: state <= S_READ; // to prevent a latch (any unknown state resets to read state) ## endcase ## end ## end ## endmodule Design Timing Summary: Power consumption: Results: A= 4 , B = 2 ALU control = 0000 (ADD): LED output = 6 ALU control = 0001 (SUB): LED output = 2 ALU control = 0010 (AND): LED output = 0 (Zero signal high) ALU control = 0011 (OR): LED output = 6 ALU control = 0100 (XOR): LED output = 6 ALU control = 0101 (SLL): LED output = 16 ALU control = 0110 (SRL): LED output = 1 ALU control = 0111 (BEQ): LED output = 2 (Zero signal low) A= 1 , B = 2 ALU control = 0001 (SUB): LED output = -1 (Sign extended)

Jazib github repo link :

https://github.com/jazibjaved0106/Labs-mj11972/tree/main/CA_Lab_06

https://github.com/Ahmaddddd7/CA_LAB/tree/main/lab6

# Lab 05 – Designing FSM Using FPGA Switches and LEDs

# Assessment Rubrics

# Points Distribution

| Task No. | LR2 Code | LR 5 Results | LR11 Design | AR7 |
| --- | --- | --- | --- | --- |
| Task 1 | - | - | /20 | /20 |
| Task 2 | /20 | /15 |  |  |
| Task 3 | /10 | /15 |  |  |
| Total Points | /80 |  |  | /20 |
| CLO Mapped | CLO 1 |  |  |  |

For description of different levels of the mapped rubrics, please refer the provided Lab Evaluation Assessment Rubrics and Affective Domain Assessment Rubrics.

| # | Assessment Elements | Level 1: Unsatisfactory Points 0-1 | Level 2: Developing Points 2 | Level 3:Good Points 3 | Level 4:Exemplary Points 4 |
| --- | --- | --- | --- | --- | --- |
| LR2 | Program/Code/ Simulation Model/ Network Model | Program/code/simulation model/network model does not implement the required functionality and has several errors. The student is not able to utilize even the basic tools of the software. | Program/code/simulation model/network model has some errors and does not produce completely accurate results. Student has limited command on the basic tools of the software. | Program/code/simulation model/network model gives correct output but not efficiently implemented or implemented by computationally complex routine. | Program/code/simulation /network model is efficiently implemented and gives correct output. Student has full command on the basic tools of the software. |
| LR5 | Results & Plots | Figures/ graphs / tables are not developed or are poorly constructed with erroneous results. Titles, captions, units are not mentioned. Data is presented in an obscure manner. | Figures, graphs and tables are drawn but contain errors. Titles, captions, units are not accurate. Data presentation is not too clear. | All figures, graphs, tables are correctly drawn but contain minor errors or some of the details are missing. | Figures / graphs / tables are correctly drawn and appropriate titles/captions and proper units are mentioned. Data presentation is systematic. |
| AR9 | Report | All the in-lab tasks are not included in report and / or the report is submitted too late. | Most of the tasks are included in report but are not well explained. All the necessary figures / plots are not included. Report is submitted after due date. | Good summary of most the in-lab tasks is included in report. The work is supported by figures and plots with explanations. The report is submitted timely. | Detailed summary of the in-lab tasks is provided. All tasks are included and explained well. Data is presented clearly including all the necessary figures, plots and tables. |